Introduction,

For this lab, we are going to estimate the state of a two wheel robots based on measurement of reflection sensor and gyro sensor. For more accuracy, we are also going to implement a extended Kalman Filter to optimize this state estimator.

Figure 1, real two-wheel paper-bot

Sensor Connections

For this lab we used IR reflection sensor and gyro scope, the IR reflection sensor will measure the IR reflection, since the reflectivity will vary based on the reflection surface, white has the maximum reflectivity while black has minimum reflectivity, since our wheel is coded by black and white stripes, this sensor can measure the how many revolutions that the wheel spin and hence measure the angular speed of the wheel. The set-up for this sensor is easy, we just connect to a digital pin of microcontroller and use the digitalRead() function to read the measurement from the sensor. However, this method cannot measure the direction of rotation, so we need other mechanism to get this data.

Figure 2Ir sensor on the robot

The gyro scope can measure the translational acceleration in three different axis and the angular velocity in three axis. This sensor use I2C protocol to communicate with microcontroller and has some bias and scaling factor need to be consider, the set-up for gyro scope is not easy,

Extended Kalman Filter Model,

Figure 3 simplified model in 2D plane

Robot’s state can be represent as a vector that has 6 variables namely, Vx, the current velocity in x-axis, Vy, the current velocity in y-axis, Vθ, the angular velocity around z-axis, X, the coordinate alone x-axis; Y, the coordinate alone y-axis; and θ the angular position about x-axis.

Current state X(k) =  VxVyVθXYθ

The control signal is the velocity of two wheels (PMW output)

Control signal uk=V1V2

Then we can build out model for theoretical state transition, out current state depends on the previous state plus the effect brought by the control signal during one time-period.

We can write the equation that

Xk=A*Xk-1+B*uk

since without input signal, the robot will stand still at current position and all velocity should be 0

Xk= 000X(k-1)Y(k-1)θ(k-1)+B*uk

So,  A = 000000000000000000000100000010000001

For B matrix we make a approximation that the time interval we use is very small, so if the velocities for two wheels are not equal, the robot supposes to drive as a curve trajectory, here we approximate this curve for a linear motion.

Vx=d2V1+V2cosθ

Vy=d2V1+V2sinθ

Vθ=V1-V2d

X(t)=Vx*t+X(k-1)

Y(t)=Vy*t+Y(k-1)

θt=Vθ*t+θ(k-1)

As we can see, this is not a linear function, so we can’t take inverse matrix of this, for that reason, we need to use Extended Kalman Filter for this lab that linearize those functions as we estimate our states.

Then we need to build sensor model for our robot. For this lab, we are using two IR reflection sensors for each wheel and a gyro scope just for angular velocity in around z-axis. So our measurement vector will contain 3 value namely Vl, velocity of left wheel, Vr, velocity of right wheel, and Vθ, which is angular velocity measured by gyro scope,

zk=VlVrVθ

For the model in Kalman filter, zk=C*xk+v(k), where z(t) is sensor measurement which is some representation of current state, v(t) is the sensor noise.

Since our sensor has only 3 measurement for the 6 states robot, we can’t conclude the matrix C in a single matrix, to simplify out problem, we assume our input signal has only 6 commands, so we can use different C matrix based on different input signal.

If the control signal is forward or backward, the car is doing only translation motion, the C matrix is

C= VxVx2+Vy2VyVx2+Vy20000VxVx2+Vy2VyVx2+Vy20000000000

This matrix may introduce a new problem, when both Vx and Vy are 0, the C matrix will have 0/0 which is undefined behavior so we modify this matrix by

C= VxVx2+Vy2+0.00001VyVx2+Vy2+0.000010000VxVx2+Vy2+0.00001VyVx2+Vy2+0.000010000000000

We add a very small number to the denominator so when Vx= Vy=0, the corresponding element in matrix is 0 and when Vx and Vy are non-zero, the influence by adding this number is neglectable.

next class of situation is when only one wheel spinning.

For the situation that left wheel stand still while right wheel move forward, the robot is turning left

C= 00d000000000001000

Where d is the distance between two wheels, which is 8.5cm.

For the situation that left wheel stand still and right wheel go backwards, the robot is backing and turning left

C= 00-d000000000001000

For the situation that left wheel go forward and right wheel stand still, the robot is turning right

C= 00000000-d000001000

For the situation that left wheel go backwards and right wheel stand still, the robot is going backward and turn right

C= 00000000d000001000

Then , the next class of situations are two wheel spinning in different direction.

For left wheel go backward while right wheel go forward.

C= 00d/200000d/2000001000

For left wheel go forward and right wheel go backward,

C= 00-d/200000d/2000001000

When the robot is not moving

C= 000000000000000000

Then we can compute the Kalman Gain for the Kalman filter, Kalman filter is the filter is using the predicted state and sensor measurements to best estimate the robot state.

The prediction model is

Xk=A*Xk-1+B*u(k)

Pk=A*Pk-1*AT

Where Xk is the estimated current state of robot, and Pk is the error factor for the model. Then we can recursively update the model and our estimation.

The update model is

Gk=PkCT(CPkCT+R)-1

Xk=Xk+Gk*(Zk-CXk)

Pk=(1-GkC)Pk

Where Gkis the Kalman Gain for our model, Xk is the updated estimation of current state. And R is covariance matrix of sensor noise. That will minimize the influence by sensor noise. That is the complete model for out Extended Kalman Filter.

Result

The user interface shows the control button and the lines under buttons give the estimated robot current states.

Control robot go straight line.

The x-coordinate increase correspondingly.

Then turn left

Then go straight again,

Both X and Y coordinates increase correspondingly

Then hit go back

Turn right then

Go forward again

Although the two wheels on the robot are always slip, and the program will not account for this situation, that cause the estimate state differ from the actual state. If operate under no slip condition, the state estimator should be able to minimize the operation and sensor noise.